Zigbee2MQTT Migration Guide

Sonoff Dongle-E v2 → Sonoff Dongle-M (EFR32MG24 / Ember)

No device re-pairing required

🧰 What You Will Need

Hardware

- Raspberry Pi 4 running Home Assistant

- Sonoff Zigbee Dongle‑E v2 (current coordinator)

- Sonoff Zigbee Dongle‑M (new coordinator)

- USB extension cable (strongly recommended to reduce interference)

Software

- Home Assistant (latest)

- Zigbee2MQTT (latest)

⚠️ Critical Rules (Read Before Starting)

- ❌ Do NOT delete Zigbee2MQTT

- ❌ Do NOT create a new Zigbee network

- ❌ Do NOT re-pair any devices

- ❌ Do NOT change PAN ID, Extended PAN ID, network key, or channel

- ✅ We are only replacing the coordinator hardware

If these rules are followed, all devices will automatically reconnect.

Step 1: Back Up Zigbee2MQTT (MANDATORY)

This backup preserves:

- Network keys

- PAN ID / Extended PAN ID

- Device IEEE addresses

- Routing information

Without this backup, re-pairing will be required.

✅ Method 1 (RECOMMENDED): Zigbee2MQTT UI Backup

This is the simplest and safest method.

- Open the Zigbee2MQTT UI

- Click Settings

- Click Tools

- Click Request Z2M backup

- Wait a few seconds

- Click Download Zigbee2MQTT backup

- Save the file to your local computer

✔ This single file contains:

- database.db

- coordinator_backup.json

- configuration.yaml

- All Zigbee network state

📌 This file alone is sufficient for a full restore.

(Optional) Alternative Backup Methods

File Editor

- Backup /config/zigbee2mqtt/

SSH

cp -r /config/zigbee2mqtt /config/zigbee2mqtt_backup

Step 2: Stop Zigbee2MQTT

- Home Assistant → Settings → Add-ons → Zigbee2MQTT

- Click Stop

- Wait until the add-on status shows Stopped

Step 3: Shut Down Home Assistant

- Go to Settings → System → Power

- Click Shut Down

- Wait until the Raspberry Pi fully powers off

Step 4: Physically Swap the Dongles

- Remove Sonoff Dongle-E v2

- Insert Sonoff Dongle-M

- Use a USB extension cable

- Power the Raspberry Pi back on

- Wait for Home Assistant to fully boot

Step 5: Identify the New Serial Port (Correct & Safe Methods)

You must use a persistent /dev/serial/by-id/ path.

❌ Never use /dev/ttyUSB0
 ✅ Always use /dev/serial/by-id/...

Method 1: Zigbee2MQTT Log

- Home Assistant → Settings → Add-ons → Zigbee2MQTT

- Click Log

- Look for a line similar to:

/dev/serial/by-id/usb-SONOFF_SONOFF_Dongle_Max_MG24_da00da6b6bfaef11b2888d256d9880ab-if00-port0

Copy this entire path.

Method 2: SSH (Very Reliable)

- Open Terminal & SSH

- Run:

ls -l /dev/serial/by-id/

- Copy the left-hand path:

/dev/serial/by-id/usb-SONOFF_SONOFF_Dongle_Max_MG24_da00da6b6bfaef11b2888d256d9880ab-if00-port0

Method 3: Home Assistant Hardware Page (GUI-Only)

- Home Assistant → Settings → Hardware

- Click All Hardware

- Scroll to the Sonoff Zigbee dongle

- Click it to expand

- Copy the device ID / path shown

✔ Home Assistant automatically provides the stable by-id path
 ✔ /dev/ttyUSB0 is excluded by design

Step 6: Update Zigbee2MQTT Configuration

- Home Assistant → Settings → Add-ons → Zigbee2MQTT

- Click Configuration

- Click Serial twisty

- Under the serial → port, enter the new serial port you copied  in step 5: /dev/serial/by-id/usb-SONOFF_SONOFF_Dongle_Max_MG24_da00da6b6bfaef11b2888d256d9880ab-if00-port0

- Click Save

✅ Correct Configuration for Sonoff Dongle-M (EFR32MG24)

serial:

port: /dev/serial/by-id/usb-SONOFF_SONOFF_Dongle_Max_MG24_da00da6b6bfaef11b2888d256d9880ab-if00-port0

adapter: ember

⚠️ Do NOT change:

- pan_id

- ext_pan_id

- network_key

- channel

Step 7: Start Zigbee2MQTT

- Go to Settings → Add-ons → Zigbee2MQTT

- Click Start

- Monitor the logs

Expected Log Lines

Using Ember adapter

Coordinator started

Zigbee network started

- Routers reconnect first

- Battery devices reconnect when they wake

- No pairing mode required

Step 8: Verify Operation

Zigbee2MQTT UI

- Devices show online

- “Last seen” timestamps update

Home Assistant

- All entities unchanged

- Automations continue working

Step 9: Optional Network Optimisation

After 12–24 hours:

- Power-cycle mains-powered routers (plugs, bulbs)

- Leave battery devices untouched

- Allow routing tables to stabilise naturally

🛑 Restore Procedure (If Needed)

- Stop Zigbee2MQTT

- Open Zigbee2MQTT UI → Settings → Tools

- Click Restore Zigbee2MQTT backup

- Upload the previously downloaded backup file

- Reinsert the original dongle if required

- Start Zigbee2MQTT

Your Zigbee network will be fully restored.

✅ Final Result

- Same Zigbee network

- Same devices & entities

- Same automations

- No re-pairing

- Upgraded, future-proof coordinator (EFR32MG24)